FLAG purification from 293T cells

Transfect 3-4 ug DNA per 10 cm dish (2x10^6 cells), 100 uL optimum, 24 uL PEI

24 hours later, split into 15 cm dish

Harvest cells into PBS at 72 hr post transfection

For RADX 5 minimum, but 10 better X10cm dish per condition yields usable protein.

Lysis

NETN lysis buffer:

15 mL 5M NaCl

10 mL Tris pH 8.0

1 mL 0.5 M EDTA

25 mL 10% NP40

447 mL ddH2O

Protease (and phosphatase if needed) inhibitor tablets

Add 5 mL to pellet (5mL per 5 dishes, so 5mL lysate total), resuspend 30 min on ice

Spin max (tabletop 3000 rpm-cold) speed 15 min- transfer to new tube

IP with agarose (magnetic) beads

25 uL beads per condition  - follow flag IP for RadX to prep resin

(Prep resin-Wash resin 2X PBS or TBS, can add BSA to block as well, 1x lysis buffer)

Add beads to lysate, cold room for 4 hours rocking (MA note- 2-2.5 hours is fine for RADX)

Elution **use magnetic flag M2 resin, no need for spin steps

Spin down and remove supernatant. Wash 1 ml lysis x3, LiCl x2, Elution Buffer x 2

Resuspend in Elution  buffer + 0.25 mg/mL (final) flag peptide

Incubate on ice for 1.5 hours, mix every 15 minutes

Spin down at max speed for 1 min -not needed if magnetic beads

Transfer supernatant to new tube- freeze -80C

RPA depletion ** (THIS IS ONLY FOR RADX PROTOCOL)

Use 20ul  rProtein G agarose bead slurry per sample (use magnetic much easier and eliminate spin steps)

Wash 1X with PBS/0/1% BSA.

Rinse twice PBS

Resuspend in 200 ul PBS + RPA antibody (9H8 Abcam) at 1:1000 dilution

Rotate 4C for 1.5 hours

Centrifuge 1000xg 2 min (magnet)

Wash 2X elution buffer

Add eluate containing purified CX from previous section. Save input sample (8 uL)

Rotate at 4C for 1 hr

Centirfuge 1000 xg for 2 min (magnet)

Collect supernatant and aliquot/western blot. This protein can now be stored at -80C and used for downstream applications.

LICL buffer (***I use lysis buffer + 0.3M LiCL instead of this**)

100 ul HEPES

1.5 mL 2M LiCl

4 mL glycerol

4 ul EDTA

5 ul 20% triton-X

10 ul DTT

10 ul PMSF

15 ul MgCl2

4.35 mL H20

Elution buffer (change this buffer as needed for what your protein prefers)

50 mM Tris pH 7.5

250 mM NaCl

10% glycerol

1 mM DTT

Add protease inhibitors

Other proteins for reference:

SMARCAL1 and ZRANB3

Lysis:

20 mM Tris (pH 7.5), 150 mM NaCl, 0.1 mM EDTA, 1 mM DTT, 0.2 mM PMSF, protease inhibitors, and 0.1% Triton X-100.

Elution:

20 mM HEPES at pH 7.6, 20% glycerol, 0.1 M KCl, 1.5 mM MgCl2, 0.2 mM EDTA, 1 mM DTT, 0.01% IGEPAL CA-630

Primpol:

Elution: 10 mM Hepes (pH 7.5), 15 mM glutathione, 1× Roche cOmplete Mini EDTA-free protease inhibitor, and 1 mM DTT]. Final: Purified proteins were dialyzed into 10 mM Hepes (pH 7.5), 50 mM NaCl, and 1× Roche cOmplete Mini EDTA-free protease inhibitor.