RADAR assay protocol to detect HMCES-DPC after CD437 treatment

1-18-22

RADAR buffer composition:

- You can use the RLT plus buffer (Qiagen Cat# 1053393). Each aliquot to use for an experiment should include 1% Sarkosyl right before the experiment.

- To make homemade RADAR buffer use the next recipe:

| Concentration | Amount | Stock |  |
| --- | --- | --- | --- |
| 6M Guanidine HCl | 114,6 g | Powder | Dissolved in 40 ml H2O |
| 0.1M Sodium acetate pH 5 | 6,7 ml | 3 M pH 5 |  |
| 20mM Tris pH 8 | 4 ml | 1 M pH 8 |  |
| 20mM EDTA pH 8 | 8 ml | 0,5 M pH 8 |  |
| Measure everything and add 20 ml of H2O |  |  |  |
| DTT | 2 g | Powder |  |
| Calibrate pH to 6.5 and make total volume up to 170 ml |  |  |  |
| 1% Sarkosyl | 10 ml | 20% |  |
| 2% Triton X-100 | 20 ml | 20% |  |
| If you filter, you create a lot of foam |  |  |  |

- Make aliquots and freeze them at -20 °C

RADAR assay

- Plate 0.6x106 HCT116 cells in 60 mm dishes.

- Next day, synchronize cells with 2mM thymidine.

- Next day, remove thymidine by washing cells with 1X PBS (twice)

- Add normal growth media and incubate cells at 37 degrees for 2 hrs.

- After those 2 hrs. incubate cells with CD437 (5uM) for 30 mins at 37 degrees. For mock treatment add DMSO.

- DNA isolation:

- Remove media and wash with 1X PBS.

- Add 1ml of RADAR buffer (RLT buffer containing 1% Sarkosyl)

- Homogenize using 1ml tips, up and down 10 times, without making bubbles, until there are no clumps.

- Put it into 1.5ml tubes.

- Let it incubate at RT for 30 mins and vortex high speed every 10 mins.

- Add 500 ul of 100% ethanol at RT.

- Invert 10 times (you should see DNA precipitation) and put samples at -20 for 5 mins.

- Centrifuge samples at full speed at 4 degrees for 20 mins.

- Remove RADAR buffer using vacuum, do not let the pellet dry.

- Add 1ml of 70% Ethanol and centrifuge high speed for 5 mins.

- Remove the ethanol using vacuum and repeat adding 1 ml 70% ethanol again.

- Centrifuge and after aspirating the ethanol, be sure there is almost no ethanol remaining with the DNA pellet. If so, there is no need to let air dry.

- Add 200ul of 8mM Na(OH) to each pellet.

* At this point you can store your samples at -20 degrees.

- HMCES blot:

- Resuspend pellets in Na(OH) using a thermomixer (800rpm, 65 degrees, 3 hrs)

- Quantify DNA concentration.

- Set up the following reaction (Rx) mix:

4ml 1X TBS, 8 ul MgCl2 (1M), 2 ul nuclease.

- Make 200ul aliquots of the Rx mix in 1.5 ml tubes and add 30ug of DNA for each sample.

- Put reaction at 37 degrees, 300rpm for 1 hr.

- Equilibrate a nitrocellulose membrane in water for 2 mins.

- Equilibrate nitrocellulose membrane with 1X TBS and set up the slot blot apparatus.

- 3 filter paper soaked in 1X TBS and membrane on top.

- Tight the device and add 200ul of 1X TBS to keep membrane wet.

- Boil samples at 95 degrees for 5 mins.

- Vortex the samples and spin them down (short).

- Apply vacuum to remove TBS.

- Turn the vacuum off and load samples.

- Load the entire samples and add 1X TBS in all the empty wells.

- Apply vacuum until all samples run through.

- Remove membrane, trim it and block with 5% milk for 1 hr. at room temperature (RT).

- Add anti-HMCES (rabbit antibody) at 1:3000 in 1% milk ON in the cold room.

- DNA blot:

- Set up the following reaction:

1 ml 6X SSC (2 tubes – 45ml: 13.5ml 20X SSC + 13.5ml H2O)

1 ug DNA

- Put reaction at 95 degrees for 10 mins and then immediately on ice.

- Equilibrate a Nylon membrane with 6X SSC and set up the slot blot apparatus.

- 3 filter paper and membrane on top.

- Add 6X SSC to keep membrane wet.

- Apply vacuum to remove SSC.

- Turn the vacuum off and load samples using same locations than HMCES blot.

- Apply vacuum until all samples run through.

- Remove membrane, trim off extra wells.

- Place membrane face up on Whatman paper soaked in 1.5M NaCl, 0.5M Na(OH) for 10 mins.

- 5.6 ml (4M NaCl) + 0.75 ml (10N Na(OH)) + 8.7 ml H2O

- Place membrane face up on Whatman paper soaked in 1.5M NaCl, 0.5M Tris pH 7.5 for 5 mins.

- 5.6 ml (4M NaCl) + 7.5 ml (1M Tris) +. 1.9 ml H2O

- Air dry membrane for 10 mins.

- UV crosslink with 1200 J/m2

- Block with 5% Milk for 1 hr. at RT.

- Incubate with mouse anti human. ssDNA 1:3000 in milk ON at the cold room.

- Blotting:

- Wash the membrane (nitrocellulose and nylon) with 1X-TBST, 8 mins/X5.

- Incubate 1 hour with second Ab (anti Rb-HRP and anti Ms-StarBright520)

- Wash the membrane (nitrocellulose and nylon) with 1X-TBST, 8 mins/X5.

- Wash finally the Nylon membrane with 1X TBS.

- Perform blotting using Femto for the HMCES blot.